CMPG 325 COMPUTER NETWORKS  |  MILESTONE 2

MOTSWEDING EVENT MANAGEMENT

Network Infrastructure Design

Milestone 2 — Client Design Review

| Project Information | Details |
| --- | --- |
| Organisation | Motsweding Event Management (Potchefstroom) |
| Industry | Entertainment |
| Project ID | CMPG325-2026-123 |
| Client ID | CLI-123 |
| Student | SEKO, OFENTSE |
| Student Number | 54461626 |
| Assigned Challenge | Router-on-a-Stick (sub-interface Inter-VLAN routing) |
| Addressing Block | 10.48.0.0/16 |
| Design Constraint | Point-of-sale systems must stay online even if the office LAN fails. |
| Change Request | CR1 — 8 additional staff in one department without redesigning the network. |
| GitHub Repository | github.com/Ofentse2212/motsweding-network-infrastructure |

Contents

1  Executive summary

2  Client requirements and the assigned challenge

3  Network design and addressing

4  Implementation

5  Verification and testing evidence

6  Troubleshooting record

7  Constraint CR2: POS resilience test

8  Limitations and scope notes

9  Evidence index and repository structure

10  Conclusion

Screenshots are numbered Figure 1 to Figure 26 in the order they appear. Section 9 maps every figure to its file name in the evidence/ folder of the GitHub repository. Three dashed boxes in Section 6 mark captures that must be pasted in from the original Packet Tracer session; they are listed in Section 9 as well.

# 1  Executive summary

This report documents the implementation, testing and troubleshooting of the Motsweding Event Management campus network, built in Cisco Packet Tracer for the CMPG 325 Milestone 2 Client Implementation Review. The network sits inside the assigned 10.48.0.0/16 block and implements the assigned Advanced challenge, Router-on-a-Stick inter-VLAN routing. Router-Core carries six VLANs (Administration, Operations, Finance, IT, Guest and Server infrastructure) as 802.1Q sub-interfaces on GigabitEthernet0/1. The seventh, the point-of-sale (POS) VLAN, is deliberately kept off that trunk and has its own routed interface, GigabitEthernet0/2, to satisfy the client constraint that the POS systems must stay online even if the office LAN fails.

Every verification test in this report ends in a pass. Switches show the seven VLANs and working 802.1Q trunks; Router-Core shows all sub-interfaces up/up with the correct gateways; hosts in the same VLAN reach each other; hosts in different VLANs reach each other through the router; the web server answers over HTTP; and a host in the IT VLAN reaches both gateways and both infrastructure servers. Change request CR1 was demonstrated by growing the Operations department to ten workstations without a new VLAN, subnet or gateway. Constraint CR2 was demonstrated by administratively shutting down Router-Core GigabitEthernet0/1, which takes the whole office LAN down, and showing that a POS terminal kept answering its gateway and a peer terminal while an office PC lost its gateway completely.

The path to those results was not smooth, and this report records it in full rather than only the final state. DHCP failed for a long time because of several overlapping faults: department-switch uplinks configured as access ports, native VLAN mismatches on the trunks, a wrong 255.0.0.0 subnet mask on the servers and duplicate DHCP leases. Packet Tracer and IOS quirks cost further time. One more point deserves to be stated plainly: my first "resilience" test shut down the access port of POS-01. That proved a single link can fail and recover, but it did not test the client constraint. It was replaced by the correct CR2 test described in Section 7, and the earlier test is kept in the record under its true name.

The report closes with the limits of what was tested (Section 8). The most important are that no default route or WAN is configured, that the CR2 result proves POS reachability to its own gateway and peers rather than to servers or the internet, and that guest wireless is built but not covered by evidence in this submission.

# 2  Client requirements and the assigned challenge

Motsweding Event Management is an entertainment-industry company in Potchefstroom. The project was assigned the address block 10.48.0.0/16, a Router-on-a-Stick networking challenge, and two requirements that shape the whole design.

The design constraint. The point-of-sale systems must stay online even if the office LAN fails. I interpret "stay online" narrowly and testably: while the office network is down, POS terminals must still be able to talk to each other and to their default gateway, and they must be reachable on their own subnet. The design does not promise that POS terminals can reach office servers or the internet during an office outage, because those live behind the very trunk that has failed. Stating this precisely matters, because overclaiming it would make the test look stronger than it is.

Change request CR1. The client signs eight additional staff in one department. Capacity must be added without redesigning the network. I treat Operations as that department, growing it from two to ten workstations.

Change request CR2 (the POS constraint under test). Because the constraint above is a hard requirement, it is tested as a failure scenario and not merely asserted from the diagram.

Milestone 2 deliverables. A working Packet Tracer file, the assigned feature implemented, testing evidence, and an updated GitHub portfolio. This report is the narrative that ties those four together.

# 3  Network design and addressing

## 3.1  Topology

The topology is a simple hierarchy built around Router-Core. Router-Edge connects to Router-Core on GigabitEthernet0/0 over the point-to-point subnet 10.48.0.0/30, where Router-Core holds 10.48.0.2. Router-Core then splits into two independent paths. GigabitEthernet0/1 runs to Core-Switch and carries the office network as an 802.1Q trunk. GigabitEthernet0/2 runs to SWITCH-POS and carries only POS traffic, untagged.

Core-Switch feeds the six office-side switches on Fa0/1 to Fa0/6: SW-ADMIN (two PCs), SW-OPS (ten PCs after CR1), SW-FIN (two PCs), SW-IT (two PCs), SW-GUEST (the guest access point AP-GUEST, with LAPTOP-GUEST-01 attached wirelessly) and SERVER-SWITCH (DHCP-SRV, FILE-SRV and WEB-SRV). SWITCH-POS serves POS-01, POS-02 and POS-03. Figure 1 and Figure 2, on the next page, show the finished topology without and with port labels.

## 3.2  Addressing plan

Each VLAN receives a /24 whose third octet equals its VLAN ID, so the VLAN can be read off any address (10.48.30.x is VLAN 30). Gateways are always .1. The routed link to Router-Edge uses a /30 because it needs only two addresses.

| VLAN | Function | Subnet | Gateway | Router interface | Attached to |
| --- | --- | --- | --- | --- | --- |
| 10 | Administration | 10.48.10.0/24 | 10.48.10.1 | Gi0/1.10 (dot1Q 10) | SW-ADMIN |
| 20 | Operations | 10.48.20.0/24 | 10.48.20.1 | Gi0/1.20 (dot1Q 20) | SW-OPS |
| 30 | Finance | 10.48.30.0/24 | 10.48.30.1 | Gi0/1.30 (dot1Q 30) | SW-FIN |
| 40 | IT Technical | 10.48.40.0/24 | 10.48.40.1 | Gi0/1.40 (dot1Q 40) | SW-IT |
| 50 | POS Network | 10.48.50.0/24 | 10.48.50.1 | Gi0/2 (routed, dedicated) | SWITCH-POS |
| 60 | Guest WiFi | 10.48.60.0/24 | 10.48.60.1 | Gi0/1.60 (dot1Q 60) | SW-GUEST |
| 70 | Server Infrastructure | 10.48.70.0/24 | 10.48.70.1 | Gi0/1.70 (dot1Q 70) | SERVER-SWITCH |
| n/a | Router link | 10.48.0.0/30 | n/a | Gi0/0 (10.48.0.2) | Router-Edge |

The three servers use static addresses in VLAN 70: DHCP-SRV 10.48.70.10, FILE-SRV 10.48.70.11 and WEB-SRV 10.48.70.12, each with mask 255.255.255.0 and gateway 10.48.70.1. All other hosts obtain their addresses by DHCP. A gateway of last resort is not configured, as discussed in Section 8.

## 3.3  Design decisions

Why Router-on-a-Stick. One physical router interface, GigabitEthernet0/1, is divided into logical sub-interfaces, one per VLAN. Each sub-interface is tagged with an 802.1Q VLAN ID and holds the gateway address for that VLAN. This gives full inter-VLAN routing with a single link to the switching fabric, which is exactly what the assigned challenge asks for. The trade-off is that all inter-VLAN traffic shares that single link, which is acceptable at this scale.

Why POS is not a sub-interface. There is no GigabitEthernet0/1.50. If the POS VLAN rode the same trunk, then shutting down or losing that trunk would take POS down with the office, which is precisely what the client forbids. Giving POS its own routed physical interface means the two failure domains are independent. A consequence of this choice is that SWITCH-POS needs no trunk at all: its uplink Gig0/1 is an ordinary access port in VLAN 50, because Router-Core Gi0/2 is a plain routed port expecting untagged frames (this is visible in Figure 14).

Why DHCP in the server VLAN with relay. A single DHCP server in VLAN 70 serves every office VLAN. Because DHCP discovery is a broadcast and broadcasts do not cross routers, each office sub-interface carries an ip helper-address pointing at DHCP-SRV so that Router-Core relays requests to it.

## 3.4  Finished topology

Figure 1. Final operational topology. Router-Edge and Router-Core at the top; the office side hangs off Core-Switch on the left (SW-ADMIN, SW-OPS with PC-OPS-01 to PC-OPS-10, SW-FIN, SW-IT, SW-GUEST with AP-GUEST and LAPTOP-GUEST-01, SERVER-SWITCH with the three servers); the independent POS path runs from Router-Core Gi0/2 to SWITCH-POS on the right. Every link shows a green (up) indicator.

Figure 2. The same topology with port labels, showing the interface on each end of every link (Router-Core Gi0/0, Gi0/1 and Gi0/2; Core-Switch Fa0/1 to Fa0/6 and Gi0/1; SW-OPS Fa0/2 to Fa0/11; SERVER-SWITCH and SWITCH-POS access ports).

# 4  Implementation

## 4.1  Physical build

The physical topology was built first, before any configuration, so that cabling could be checked on its own. Figure 3 shows that stage: every device is placed and cabled, and the router-facing links still show red indicators because the router interfaces had not yet been enabled. At this point the guest laptop was still cabled to SW-GUEST by Ethernet; it was later moved to the wireless adapter so that it associates with AP-GUEST, as the finished topology in Figure 1 shows with the dotted wireless link.

Figure 3. Initial physical build before configuration. All cabling is in place; the red indicators on the router links simply mean the interfaces were not yet enabled or addressed.

## 4.2  Layer 2: VLANs and trunks

Core-Switch holds all seven VLANs. VLAN names were chosen to match the client departments (Administration, Operations, Finance, IT_Technical, POS_Network, Guest_WiFi, Server_Infrastructure). Ports Fa0/1 to Fa0/6 toward the access switches, and Gig0/1 toward Router-Core, are 802.1Q trunks. The Gig0/1 trunk allows VLANs 10, 20, 30, 40, 60 and 70 only; VLAN 50 is left off on purpose because POS never uses that link. Every trunk uses the default native VLAN, 1, on both ends, which avoids the mismatch warnings described in Section 6.

Core-Switch(config)# vlan 10

Core-Switch(config-vlan)# name Administration

... same pattern for VLANs 20 Operations, 30 Finance, 40 IT_Technical,

50 POS_Network, 60 Guest_WiFi, 70 Server_Infrastructure

Core-Switch(config)# interface range FastEthernet0/1 - 6

Core-Switch(config-if-range)# switchport mode trunk

Core-Switch(config-if-range)# switchport trunk native vlan 1

Core-Switch(config-if-range)# switchport trunk allowed vlan 10,20,30,40,50,60,70

Core-Switch(config)# interface GigabitEthernet0/1

Core-Switch(config-if)# description Trunk to Router-Core (RoAS)

Core-Switch(config-if)# switchport mode trunk

Core-Switch(config-if)# switchport trunk allowed vlan 10,20,30,40,60,70

Core-Switch# write memory

Each departmental access switch follows one pattern. Fa0/1 is the trunk to Core-Switch, and the remaining ports are access ports in the department VLAN. SW-FIN is shown as the example; SW-ADMIN, SW-OPS and SW-IT are identical apart from the VLAN number (10, 20 and 40) and name.

SW-FIN(config)# vlan 30

SW-FIN(config-vlan)# name FINANCE

SW-FIN(config)# interface FastEthernet0/1

SW-FIN(config-if)# description Trunk to Core-Switch

SW-FIN(config-if)# switchport mode trunk

SW-FIN(config-if)# switchport trunk native vlan 1

SW-FIN(config-if)# switchport trunk allowed vlan 10,20,30,40,50,60,70

SW-FIN(config)# interface range FastEthernet0/2 - 24

SW-FIN(config-if-range)# switchport mode access

SW-FIN(config-if-range)# switchport access vlan 30

SW-FIN(config-if-range)# no shutdown

SW-FIN# write memory

SERVER-SWITCH uses a trunk on Fa0/1 toward Core-Switch (native VLAN 1, VLAN 70 allowed) and access ports in VLAN 70 for the three servers. SWITCH-POS is the exception described in Section 3.3: its three POS ports and its Gig0/1 uplink are all access ports in VLAN 50.

SWITCH-POS(config)# vlan 50

SWITCH-POS(config-vlan)# name POS_Network

SWITCH-POS(config)# interface range FastEthernet0/1 - 3

SWITCH-POS(config-if-range)# switchport mode access

SWITCH-POS(config-if-range)# switchport access vlan 50

SWITCH-POS(config)# interface GigabitEthernet0/1

SWITCH-POS(config-if)# description Dedicated uplink to Router-Core Gi0/2

SWITCH-POS(config-if)# switchport mode access

SWITCH-POS(config-if)# switchport access vlan 50

SWITCH-POS# write memory

## 4.3  Layer 3: Router-on-a-Stick on Router-Core

The physical interface GigabitEthernet0/1 has no IP address of its own; it simply has to be up. Each VLAN gets a sub-interface whose encapsulation dot1Q value matches the VLAN ID, and whose IP address is the gateway for that VLAN. The five sub-interfaces whose hosts use DHCP also carry the relay address of DHCP-SRV. Gi0/2 is a plain routed interface for POS.

Router-Core(config)# interface GigabitEthernet0/0

Router-Core(config-if)# ip address 10.48.0.2 255.255.255.252

Router-Core(config)# interface GigabitEthernet0/1

Router-Core(config-if)# no shutdown

Router-Core(config)# interface GigabitEthernet0/1.10

Router-Core(config-subif)# encapsulation dot1Q 10

Router-Core(config-subif)# ip address 10.48.10.1 255.255.255.0

Router-Core(config-subif)# ip helper-address 10.48.70.10

... .20 (10.48.20.1), .30 (10.48.30.1), .40 (10.48.40.1) and .60 (10.48.60.1)

follow the same pattern, each with encapsulation dot1Q <VLAN> and the helper

Router-Core(config)# interface GigabitEthernet0/1.70

Router-Core(config-subif)# encapsulation dot1Q 70

Router-Core(config-subif)# ip address 10.48.70.1 255.255.255.0

Router-Core(config)# interface GigabitEthernet0/2

Router-Core(config-if)# ip address 10.48.50.1 255.255.255.0

Router-Core(config-if)# no shutdown

Router-Core# write memory

The complete running configurations of Router-Core, Core-Switch, SWITCH-POS, SW-FIN and SERVER-SWITCH are stored in the configs/ folder of the repository.

## 4.4  DHCP and server services

DHCP-SRV (10.48.70.10) runs the DHCP service with one pool per DHCP-served VLAN: Administration, Operations, Finance, IT and Guest. Every pool hands out addresses starting at .10 of its subnet with mask 255.255.255.0, the VLAN gateway as default gateway, and 10.48.70.10 as DNS server. WEB-SRV (10.48.70.12) serves the default Packet Tracer web page over HTTP, and FILE-SRV (10.48.70.11) is addressed and reserved for file and TFTP services. Because DHCP replies are relayed by Router-Core, a PC in any office VLAN can pick up an address from the central server, as the first DHCP lease of the project (Finance, 10.48.30.10) showed.

## 4.5  Change request CR1: Operations expansion

Operations began with PC-OPS-01 and PC-OPS-02. For CR1, eight more workstations (PC-OPS-03 to PC-OPS-10) were added to SW-OPS, giving ten Operations PCs on access ports Fa0/2 to Fa0/11. The access-port range on SW-OPS was already assigned to VLAN 20, so the new machines needed only a cable and a DHCP request. There is no new VLAN, no new subnet, no new gateway and no router change: the new hosts simply take the next free addresses in 10.48.20.0/24. That is what "capacity added without redesign" means in practice, and Figure 2 shows the ten PCs fanned out from SW-OPS.

# 5  Verification and testing evidence

The tests below follow the layers of the network from the bottom up: VLANs and trunks, then routing, then host-to-host reachability, then services, then the scalability test. Where the first attempt of a test failed before succeeding, the screenshot shows both and Section 6 explains why.

## 5.1  VLAN and trunk verification

On SW-FIN, show vlan brief shows VLAN 30 (FINANCE) active on ports Fa0/2 to Fa0/24, and show interfaces trunk shows Fa0/1 trunking with 802.1Q encapsulation, native VLAN 1 and VLANs 10 to 70 allowed. The line "allowed and active in management domain" lists only VLAN 30, which is correct: an access switch only holds the VLANs created on it, and SW-FIN has only Finance.

Figure 4. SW-FIN: VLAN 30 (FINANCE) on Fa0/2 to Fa0/24 and a single 802.1Q trunk on Fa0/1 with native VLAN 1 and VLANs 10,20,30,40,50,60,70 allowed.

On Core-Switch, show vlan brief lists all seven VLANs and show interfaces trunk shows Fa0/1 to Fa0/6 and Gig0/1 trunking. Gig0/1, the link to Router-Core, allows 10,20,30,40,60,70 and omits VLAN 50 as designed. This capture was taken before the final native VLAN fix and still contains %CDP-4-NATIVE_VLAN_MISMATCH messages for Fa0/6 and SERVER-SWITCH; they are the evidence for the fault described in Section 6.5, and the post-fix capture appears there.

Figure 5. Core-Switch: seven VLANs and trunks on Fa0/1 to Fa0/6 and Gig0/1. The CDP native VLAN mismatch messages for Fa0/6 and SERVER-SWITCH Fa0/1 are visible and are the "before" evidence for Section 6.5.

## 5.2  Router-on-a-Stick verification

On Router-Core, show ip interface brief shows Gi0/0 (10.48.0.2), the unaddressed physical Gi0/1, sub-interfaces Gi0/1.10, .20, .30, .40, .60 and .70 with gateways 10.48.10.1 to 10.48.70.1, and Gi0/2 at 10.48.50.1, all up/up. Vlan1 is administratively down, which is expected on a router with no VLAN interface in use. The routing table lists each subnet as directly connected on the matching sub-interface, with POS (10.48.50.0/24) connected on the separate interface Gi0/2. The capture ends at the 10.48.50.0/24 entry; the table header reports 16 subnets, which accounts exactly for the connected and local entries of the remaining 10.48.50.1, 10.48.60.0 and 10.48.70.0 networks that were not scrolled into view.

Figure 6. Router-Core: sub-interfaces Gi0/1.10 to Gi0/1.70 and Gi0/2 up/up with the correct gateways, and the connected routes for each subnet. No gateway of last resort is set.

## 5.3  Same-VLAN and inter-VLAN connectivity

PC-FIN-01 pinged PC-FIN-02 (10.48.30.11) in the same VLAN. The first attempt timed out on all four packets and the second returned four replies at TTL 128, the unchanged TTL of a host on the same segment. I attribute the first-attempt failure to the duplicate DHCP lease problem described in Section 6.3.

Figure 7. Same-VLAN test from PC-FIN-01 to 10.48.30.11: an initial failed attempt followed by 4/4 replies, 0% loss, TTL=128.

PC-FIN-01 then pinged 10.48.10.10 in a different VLAN. The first packet timed out while the router and host resolved each other with ARP; the remaining packets, and the whole second run, replied with TTL 127. TTL 127 is the proof that the packet crossed exactly one router: the destination decremented a starting value of 128 once. The address 10.48.10.10 belongs to PC-ADMIN-02, not PC-ADMIN-01, because DHCP hands out addresses in order of request (see Section 6.3).

Figure 8. Inter-VLAN test from PC-FIN-01 (VLAN 30) to 10.48.10.10 (VLAN 10) through Router-Core: one ARP timeout, then replies with TTL=127; second run 4/4, 0% loss.

## 5.4  Service access and connectivity matrix

From an office workstation, the browser opened http://10.48.70.12 and loaded the default Packet Tracer page from WEB-SRV, proving routed HTTP access to the server VLAN.

Figure 9. HTTP test: the web page served by WEB-SRV (10.48.70.12) rendered in an office workstation browser.

The connectivity matrix is run from PC-IT-01 against four targets: its own gateway (10.48.40.1), the server VLAN gateway (10.48.70.1), DHCP-SRV (10.48.70.10) and WEB-SRV (10.48.70.12). The first capture, Figure 10, shows the work in progress, including a mistake (several commands on one line, which the Packet Tracer prompt rejects) and DHCP-SRV answering unreliably on its first attempts. The final capture, Figure 11, shows all four targets at 0% loss.

| Figure 10. PC-IT-01, first attempts: gateway pings succeed; the combined-command line returns "Invalid Command"; 10.48.70.10 first shows 100% loss, then 25% loss. | Figure 11. PC-IT-01 final matrix: 10.48.40.1, 10.48.70.1, 10.48.70.10 and 10.48.70.12 all reply, 4/4, 0% loss. |
| --- | --- |

## 5.5  CR1 scalability evidence

PC-OPS-10, the last of the eight added workstations, obtained an address in 10.48.20.0/24 and pinged WEB-SRV (10.48.70.12) across subnets. The first packet timed out for ARP and the second run replied 4/4, TTL 127. This shows that a host added without any configuration change on the router or core is fully functional on its existing VLAN.

Figure 12. CR1: PC-OPS-10 reaching WEB-SRV across VLANs: one ARP timeout, then replies at TTL=127; second run 4/4, 0% loss.

# 6  Troubleshooting record

This section is the chronological record of what went wrong and how each problem was found and fixed. Most of the effort of Milestone 2 went here, and almost every fault was a configuration mismatch between two ends of a link rather than a missing feature.

## 6.1  Packet Tracer Note tool triggered by a keyboard shortcut

While fast-forwarding time to let Spanning Tree settle, I pressed the Fast Forward shortcut (Alt+N, Option+N on the Mac) too many times. On macOS that shortcut places a Note in the workspace, so afterwards every click on the canvas opened a text box asking for a note. Nothing was broken. Pressing Esc, or clicking the Select tool, returned the cursor to normal. From then on I used the on-screen Fast Forward button instead of the keyboard shortcut.

## 6.2  DHCP failure: a chain of faults, not a single one

The first symptom was that PC-FIN-01 fell back to an automatic private address (169.254.x.x), which means its DHCP request never received an answer. A DHCP request is a chain: PC to access port, access switch, trunk, core switch, trunk, router sub-interface, relay, server, and back. Any one broken link looks identical from the PC. I found four separate faults on that chain, and fixing only one at a time made it look as if nothing had changed.

Fault A: missing VLANs and unassigned ports. My first assumption was that the VLANs did not exist on the switches, since a switch drops frames tagged for a VLAN it does not know. I created the VLANs on Core-Switch and the access switch and assigned the PC ports. DHCP still failed. At that point I paused DHCP deliberately so that the other phases of the build plan (switching verification, Router-on-a-Stick and addressing) could move forward, and then returned to it as its own phase with a systematic checklist of the whole chain. Those other phases later verified correctly (Section 5).

Fault B: uplinks configured as access ports. When I configured SW-IT, I applied the access-port range to Fa0/1, but Fa0/1 is the cable to Core-Switch. Spanning Tree immediately complained:

%SPANTREE-2-RECV_PVID_ERR: Received 802.1Q BPDU on non trunk FastEthernet0/1 VLAN1.

%SPANTREE-2-BLOCK_PVID_LOCAL: Blocking FastEthernet0/1 on VLAN0001. Inconsistent port type.

Core-Switch was sending trunk frames into an access port, so Spanning Tree blocked the port, and with the only uplink blocked, no DHCP traffic could leave the switch. The fix on every department switch was to make Fa0/1 a trunk and move the PC ports to Fa0/2 and above.

Fault C: native VLAN mismatches on the department uplinks. Once the ports were trunks, the consoles of SW-IT, SW-ADMIN, SW-OPS and SW-FIN repeated native VLAN warnings, for example:

%CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on FastEthernet0/1 (40),

with Core-Switch FastEthernet0/4 (1).

The department-switch ports still carried their access VLAN (40, 10, 20, 30) as the untagged VLAN, while the Core-Switch ends used native VLAN 1. I set both ends of every uplink to switchport trunk native vlan 1 with VLANs 10 to 70 allowed, and the warnings stopped.

Fault D: the wrong subnet mask on the servers. The server screenshots showed a subnet mask of 255.0.0.0 on DHCP-SRV, FILE-SRV and WEB-SRV, which is the default Class A mask Packet Tracer fills in. With that mask a server believes the whole of 10.0.0.0/8 is local, so it ARPs for off-subnet addresses instead of sending them to its gateway and cross-VLAN communication breaks. I changed all three to 255.255.255.0 with gateway 10.48.70.1, and also checked SERVER-SWITCH so that Fa0/2 to Fa0/4 were access ports in VLAN 70.

After all four faults were fixed, PC-FIN-01 requested an address again and received 10.48.30.10 with gateway 10.48.30.1 and DNS server 10.48.70.10. That was the first end-to-end DHCP success of the project.

## 6.3  Duplicate DHCP leases and the order of addresses

Even after DHCP worked, pings between PC-FIN-01 and PC-FIN-02 failed on the first run (the first capture in Figure 7), and several PCs appeared to share the same address. The most likely cause was stale lease bindings on DHCP-SRV left over from the earlier failed attempts; I did not isolate it further, because clearing the bindings resolved it. I cleared them by turning the DHCP service off and on again on DHCP-SRV, then switching each PC from Static back to DHCP so that it sent a fresh request. After that every PC held a distinct address, and the Finance ping succeeded.

One side-effect is worth recording so that it does not look like a mistake: PC-ADMIN-02 holds 10.48.10.10 and PC-ADMIN-01 holds .11, and the same swap occurred in Operations. DHCP hands out the next free address to whichever PC asks first, so the number does not follow the host name. It has no effect on routing, but it explains why a ping to 10.48.10.10 reached PC-ADMIN-02.

## 6.4  First-attempt ping failures, ARP, and Packet Tracer command-line limits

A first ping between hosts that have never talked often loses its first packet, because the sender must ARP for the target first and Packet Tracer times the packet out while it waits. This explains the single lost packet in Figure 8 and Figure 12. It does not explain whole runs of four timeouts, which indicate a real fault, as in Figure 7 (duplicate leases) and the first DHCP-SRV attempts in Figure 10. For DHCP-SRV I rechecked that its port was in VLAN 70, that its static address and gateway were correct, and let the topology settle; it then answered every packet (Figure 11).

Three Packet Tracer command-prompt limits also cost time. The PC prompt rejects several commands on one line ("Invalid Command", visible in Figure 10), does not support the -t continuous-ping flag (Figure 13), and does not accept pasted text in its terminal, so each ping has to be typed on its own line.

## 6.5  Native VLAN mismatch between Core-Switch Fa0/6 and SERVER-SWITCH Fa0/1

One mismatch survived the department-switch fixes. Core-Switch kept logging:

%CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on FastEthernet0/6 (1),

with Server-Switch FastEthernet0/1 (70).

The wording of the message identifies which side is wrong. The local port, Core-Switch Fa0/6, reports native VLAN 1; the neighbour, SERVER-SWITCH Fa0/1, reports native VLAN 70. Core-Switch was therefore already correct and the fix belonged on SERVER-SWITCH, where I set switchport trunk native vlan 1 on Fa0/1 so that both ends use VLAN 1. (A mismatch matters because untagged frames on a trunk are placed in the native VLAN, so unequal values can leak or mislabel traffic between VLANs.) The "before" state is visible in Figure 5. The captures below show the corrected state.

SERVER-SWITCH(config)# interface FastEthernet0/1

SERVER-SWITCH(config-if)# switchport mode trunk

SERVER-SWITCH(config-if)# switchport trunk native vlan 1

SERVER-SWITCH(config-if)# switchport trunk allowed vlan 70

SERVER-SWITCH# write memory

## 6.6  The first POS resilience test, and why it was the wrong test

The first attempt at the resilience requirement was an individual-link test. I confirmed that POS-01 could reach the gateway 10.48.50.1, then shut down FastEthernet0/1 on SWITCH-POS, which is the access port that POS-01 plugs into. Pinging then failed as expected. Figure 14 captures the moment: the top part shows show vlan brief on SWITCH-POS (VLAN 50 holds Fa0/1, Fa0/2, Fa0/3 and Gig0/1, so the VLAN configuration was never the fault), and the bottom part shows the shutdown command being entered.

Figure 13. POS-01 after the port was shut down: "ping 10.48.50.1 -t" rejected as Invalid Command, then 100% loss.

Figure 14. SWITCH-POS: VLAN 50 (POS_Network) on Fa0/1, Fa0/2, Fa0/3 and Gig0/1 (top); the deliberate shutdown of FastEthernet0/1 (bottom).

Figure 15. POS-01 repeated pings to 10.48.50.1 while the port was down: 100% loss each time.

I restored the port with no shutdown and saved the configuration (Figure 16). The topology showed every POS-side link green again (Figure 17), but POS-01 still failed several attempts in a row (Figure 18). It answered only after I re-enabled DHCP on POS-01 so that it requested a fresh address, after which the gateway replied 4/4 (Figure 19).

Figure 16. SWITCH-POS CLI: shutdown of FastEthernet0/1, then no shutdown, then write memory ([OK]); the log shows the interface changing to administratively down and back up.

Figure 17. SWITCH-POS after restoration: the uplink Gig0/1 and the POS ports Fa0/1, Fa0/2 and Fa0/3 all show green link indicators.

Figure 18. POS-01 ping history: the invalid -t flag, four consecutive 100%-loss runs, then a successful run once its address was refreshed.

Figure 19. POS-01 to 10.48.50.1 after recovery: 4/4 replies, TTL=255, 0% loss.

What this test proved, and what it did not. It proved that a single POS access link can fail and be restored, and it exposed that a terminal may need its address refreshed after a link bounce. It did not prove the client constraint, because the office LAN stayed up the whole time, and it shut down the wrong interface for that purpose. In the report and the repository it is therefore called the individual POS link failure and recovery test, and not the CR2 test. The real constraint test follows.

# 7  Constraint CR2: POS resilience test

## 7.1  What is being proved

The constraint says POS must stay online if the office LAN fails. The design answer is that the office LAN depends on Router-Core Gi0/1 and everything behind it, while POS depends only on Router-Core Gi0/2 and SWITCH-POS. The test therefore removes the office LAN by shutting down Gi0/1 and checks two things at once: POS continues to work, and the office really is down. The second check matters, because evidence that POS works is only meaningful if the office failure actually happened.

## 7.2  Procedure and evidence

Step 1: baseline. With everything healthy, POS-01 pinged its gateway 10.48.50.1 and the peer terminal 10.48.50.12 (POS-03). The gateway answered 4/4. The first ping to .12 timed out entirely and the second returned 4/4 at TTL 128; the failed run was most likely lost to initial address resolution, because the destination was not yet in the ARP cache.

Figure 20. Baseline before the failure: POS-01 to 10.48.50.1 4/4 replies; to 10.48.50.12 one failed run, then 4/4 replies.

Step 2: a privilege-mode mistake. Router-Core was at the user prompt (Router-Core>), so configure terminal and interface g0/1 were rejected with "Invalid input detected at '^' marker", and shutdown was treated as a host name that IOS tried to resolve through DNS ("Translating 'shutdown'... domain server"). Configuration commands only exist in privileged mode; I typed enable first.

Figure 21. Router-Core at the user EXEC prompt (>): configure terminal, interface g0/1 and shutdown all rejected or misread as host names.

Step 3: take the office LAN down. From privileged mode I shut down only Gi0/1. The log shows Gi0/1 going administratively down and all six sub-interfaces (.10, .20, .30, .40, .60, .70) following it. Gi0/2 does not appear, because it was deliberately left up.

Router-Core> enable

Router-Core# configure terminal

Router-Core(config)# interface g0/1

Router-Core(config-if)# shutdown

Figure 22. Router-Core: Gi0/1 administratively down, with sub-interfaces Gi0/1.10, .20, .30, .40, .60 and .70 all changing to down. Gi0/2 is untouched.

Step 4: POS keeps working. While Gi0/1 was down, POS-01 pinged 10.48.50.1 and received four replies at TTL 255 (the router itself, reached through Gi0/2). It then pinged 10.48.50.12 and received four replies at TTL 128. One command was typed without the word "ping" and rejected as Invalid Command; the retyped command succeeded. The topology confirms the picture: the link between Router-Core and Core-Switch is red while the Router-Core to SWITCH-POS link is green.

Figure 23. During the office outage: POS-01 gets 4/4 replies from 10.48.50.1 (TTL 255) and 4/4 from 10.48.50.12 (TTL 128), 0% loss.

Figure 24. Topology during the outage: red indicators on Router-Core Gi0/1 and Core-Switch Gi0/1 (the office trunk); the POS link and all POS ports remain green.

Step 5: the office is genuinely down. From the Finance workstation PC-FIN-01, a ping to its own gateway 10.48.30.1 failed on all four packets, 100% loss. The sub-interface that provides that gateway is down, so this is the contrast that makes the POS result meaningful.

Figure 25. During the outage: PC-FIN-01 cannot reach its gateway 10.48.30.1, 4/4 timeouts, 100% loss.

Step 6: restore. I re-enabled Gi0/1 with no shutdown and ran show ip interface brief. The log shows Gi0/1 coming up and the line protocol of all six sub-interfaces returning to up, and the table shows Gi0/1 and every sub-interface up/up, with Gi0/2 still up/up at 10.48.50.1.

Figure 26. Restoration: Gi0/1 and sub-interfaces .10 to .70 back to up/up; Gi0/2 unchanged at 10.48.50.1.

## 7.3  Result and how it should be read

During a complete office LAN outage, POS-01 kept 0% loss to its gateway through Gi0/2 and to a peer terminal, while an office PC lost its gateway entirely. The gateway ping crosses the dedicated Gi0/2 link and proves the POS path through the router; the peer ping stays inside SWITCH-POS because both terminals are in the same VLAN, so it proves local POS switching rather than the router. Together they show the POS network is functional and independent of the office trunk. The result is stated as: POS retains connectivity to its dedicated gateway and peers when the office trunk fails. It is deliberately not stated as POS having full connectivity during an office failure, because during that failure it cannot reach office VLANs or the servers behind the shut-down trunk.

# 8  Limitations and scope notes

- No default route or WAN. Router-Edge is cabled and addressed on 10.48.0.0/30, but external connectivity was out of scope for this milestone, so Router-Core shows "Gateway of last resort is not set" and nothing is claimed about internet access.

- CR2 proves reachability to the POS gateway and peers only. POS cannot reach servers or office VLANs while the office trunk is down. The POS terminals needed a refreshed address after a link bounce (Section 6.6); if a terminal needs a new lease while Gi0/1 is down, the request cannot reach DHCP-SRV, which sits in VLAN 70 behind that trunk. A DHCP pool on Router-Core for 10.48.50.0/24, or static POS addresses, would remove that dependency and is the recommended hardening step.

- Single points of failure. Router-Core, Core-Switch and the Gi0/1 trunk are each a single path for the office LAN, and all inter-VLAN traffic shares one link. This is a deliberate trade-off of the Router-on-a-Stick design at this scale.

- Guest wireless is built but not evidenced. AP-GUEST is cabled to SW-GUEST and LAPTOP-GUEST-01 is attached wirelessly, as Figure 1 shows, but this submission includes no capture of a guest client obtaining a 10.48.60.x address or reaching its gateway, so it is not claimed as verified.

- Screenshot limits. The route-table capture in Figure 6 is cut off before its last entries (explained in Section 5.2).

# 9  Evidence index and repository structure

All screenshots are stored in the evidence/ folder of the GitHub repository https://github.com/Ofentse2212/motsweding-network-infrastructure under the file names below.

| Fig | Repository file | What it shows |
| --- | --- | --- |
| 1 | evidence/fig01-final-topology.png | Final operational topology |
| 2 | evidence/fig02-port-labelled-topology.png | Topology with port labels |
| 3 | evidence/fig03-initial-physical-build.png | Initial physical build, before configuration |
| 4 | evidence/fig04-sw-fin-vlan-trunk.png | SW-FIN VLAN and trunk verification |
| 5 | evidence/fig05-core-switch-vlan-trunk-cdp.png | Core-Switch VLANs and trunks, CDP mismatch visible |
| 6 | evidence/fig06-router-core-roas-routes.png | Router-Core RoAS interfaces and routes |
| 7 | evidence/fig07-same-vlan-ping.png | Same-VLAN ping, PC-FIN-01 |
| 8 | evidence/fig08-inter-vlan-ping.png | Inter-VLAN ping, PC-FIN-01 to VLAN 10 |
| 9 | evidence/fig09-web-server-access.png | HTTP access to WEB-SRV |
| 10 | evidence/fig10-pc-it-01-first-attempts.png | PC-IT-01 first matrix attempts |
| 11 | evidence/fig11-connectivity-matrix.png | PC-IT-01 final connectivity matrix |
| 12 | evidence/fig12-cr1-pc-ops-10.png | CR1: PC-OPS-10 to WEB-SRV |
| 13 | evidence/fig13-pos-ping-t-invalid.png | POS-01 ping -t invalid, 100% loss |
| 14 | evidence/fig14-switch-pos-vlan50-shutdown.png | SWITCH-POS VLAN 50 membership and deliberate shutdown |
| 15 | evidence/fig15-pos-repeated-timeouts.png | POS-01 repeated timeouts |
| 16 | evidence/fig16-switch-pos-recovery-cli.png | SWITCH-POS recovery CLI |
| 17 | evidence/fig17-switch-pos-links-green.png | SWITCH-POS links green |
| 18 | evidence/fig18-pos-ping-history.png | POS-01 ping history |
| 19 | evidence/fig19-pos-gateway-reply.png | POS-01 gateway reply after recovery |
| 20 | evidence/fig20-cr2-baseline-pos.png | CR2 baseline |
| 21 | evidence/fig21-cr2-exec-mode-errors.png | Router-Core EXEC mode errors |
| 22 | evidence/fig22-cr2-g0-1-shutdown.png | Router-Core Gi0/1 shutdown |
| 23 | evidence/fig23-cr2-pos-still-online.png | POS still online during outage |
| 24 | evidence/fig24-cr2-topology-outage.png | Topology during outage |
| 25 | evidence/fig25-cr2-office-pc-fails.png | Office PC fails during outage |
| 26 | evidence/fig26-cr2-restoration.png | Router-Core restoration |
| 27 | evidence/fig27-dhcp-ipconfig.png | DHCP lease on PC-FIN-01 (paste from original session) |
| 28 | evidence/fig28-server-switch-trunk.png | SERVER-SWITCH trunk after native VLAN fix (paste) |
| 29 | evidence/fig29-core-switch-trunk-fixed.png | Core-Switch trunk after native VLAN fix (paste) |

Recommended repository layout:

motsweding-network-infrastructure/

├── README.md

├── docs/

│   ├── Milestone2_Implementation_Report.docx

│   ├── configuration.md

│   ├── testing.md

│   └── troubleshooting.md

├── packet-tracer/

│   └── motsweding-network.pkt

├── configs/

│   ├── Router-Core.txt

│   ├── Core-Switch.txt

│   ├── SWITCH-POS.txt

│   ├── SW-FIN.txt

│   └── SERVER-SWITCH.txt

└── evidence/

└── fig01-... to fig29-... (.png)

# 10  Conclusion

The Motsweding network is implemented as designed. Router-on-a-Stick on Router-Core Gi0/1 routes six VLANs through 802.1Q sub-interfaces, a central DHCP server serves the office VLANs through relay, the web server answers across subnets, and the Operations department grew from two to ten workstations without any redesign. The client constraint was tested as a real failure: with the office trunk shut down, POS kept its gateway and peers while the office lost its gateway, and recovery was verified.

Equally important is what the evidence shows about the process. The faults that took longest were configuration mismatches between the two ends of a link (access against trunk, native VLAN 70 against 1, a 255.0.0.0 mask against a /24), and the most valuable correction was recognising that my first resilience test did not test the requirement. The final report keeps both the successes and these corrections, and it states the limits of the CR2 claim as carefully as the claim itself.